# PRD — IoT Smart Climate Controller

## 1. Загальний опис продукту

IoT Smart Climate Controller — IoT-пристрій на базі ESP32 для автоматичного підтримання заданої температури та вологості в приміщенні.

Пристрій керує:

- кондиціонером у режимах HEAT, COOL, DRY, OFF;

- системою зволоження;

- автоматичним наповненням резервуара для води.

Система підтримує автоматичний і ручний режими, конфігуровані пороги, аварійні стани та віддалений моніторинг і керування.

## 2. Призначення продукту

Основне призначення системи — автоматично підтримувати комфортний мікроклімат у приміщенні без постійного втручання користувача.

Пристрій контролює температуру та вологість, керує виконавчими механізмами, дозволяє змінювати параметри роботи віддалено та використовується як цільова система для HIL/E2E автоматизованого тестування.

## 3. Технічний стек

- Main Controller:				ESP32

- Firmware:					MicroPython

- Temperature / Humidity Sensor:		SHT31

- Sensor Interface:				I²C

- Water Level Sensors:				LOW Level Sensor, FULL Level Sensor

- Water Sensor Interface:			Digital GPIO

- AC Control:					Infrared (IR)

- Pump / Valve Control:				GPIO + Relay/MOSFET Driver

- Wireless:					Wi-Fi 2.4 GHz

- IoT Protocol:					MQTT

- MQTT Client:					MicroPython umqtt

- Configuration Storage:			ESP32 Non-Volatile Storage

- Diagnostics:					UART / USB Serial

## 4. Functional Requirements

### 4.1 Default Configuration

- Min Temperature:				20°C

- Max Temperature:				25°C

- Target Temperature:				22.5°C

- Min Humidity:					40%RH

- Max Humidity:					60%RH

- Target Humidity:				50%RH

- Measurement Interval:			5 s

- WARNING Low Temperature:			<16°C

- WARNING High Temperature:		>30°C

- CRITICAL Low Temperature:			<10°C

- CRITICAL High Temperature:			>35°C

- WARNING Low Humidity:			<30%RH

- WARNING High Humidity:			>70%RH

- CRITICAL Low Humidity:			<20%RH

- CRITICAL High Humidity:			>85%RH

- Max Refill Time:				120 s

Target values are calculated automatically:

Target Temperature = (Min Temperature + Max Temperature) / 2

Target Humidity = (Min Humidity + Max Humidity) / 2

### 4.2 Climate Control Rules

Temperature and humidity regulation are handled as two related but partially independent control channels.

#### Temperature Control

The air conditioner is responsible for temperature regulation.

HEAT

- Active only in AUTO.

- Starts when Temperature < Min Temperature.

- Continues until Temperature >= Target Temperature.

- Default behavior: <20°C → HEAT, >=22.5°C → HEAT OFF.

COOL

- Active only in AUTO.

- Starts when Temperature > Max Temperature.

- Continues until Temperature <= Target Temperature.

- Default behavior: >25°C → COOL, <=22.5°C → COOL OFF.

The air conditioner cannot operate in HEAT, COOL, and DRY simultaneously.

#### Humidification Control

The humidifier is controlled independently from HEAT and COOL.

HUMIDIFY

- Active only in AUTO.

- Starts when Humidity < Min Humidity.

- Continues until Humidity >= Target Humidity.

- Default behavior: <40%RH → HUMIDIFY, >=50%RH → HUMIDIFY OFF.

- May operate simultaneously with HEAT.

- May operate simultaneously with COOL.

- Must stop immediately when:

  - WATER_LOW is active;

  - reservoir refill is active;

  - Safe State is active.

#### Drying Control

DRY

- Active only in AUTO.

- Starts when Humidity > Max Humidity and neither HEAT nor COOL is required.

- Continues until Humidity <= Target Humidity.

- Default behavior: >60%RH → DRY, <=50%RH → DRY OFF.

- DRY and HUMIDIFY must never operate simultaneously.

- If temperature leaves the allowed range while DRY is active:

  - stop DRY;

  - re-evaluate temperature;

  - start HEAT or COOL if required.

#### Valid Combined States

The following combinations are allowed:

- HEAT + HUMIDIFY

- COOL + HUMIDIFY

- HEAT

- COOL

- DRY

- HUMIDIFY

- IDLE

The following combination is not allowed:

- DRY + HUMIDIFY

#### Priority Rules

- Safe State has the highest priority.

- When selecting an air-conditioner mode, temperature regulation (HEAT / COOL) has priority over DRY.

- HUMIDIFY is controlled independently and may run together with HEAT or COOL.

- DRY and HUMIDIFY are mutually exclusive.

- Reservoir refill may operate independently from climate regulation but blocks the humidifier while refill is active.

### FR-01 — Temperature and Humidity Monitoring

The system shall read temperature and humidity from SHT31.

- Default measurement interval: 5 seconds.

- Configurable range: 1–60 seconds.

- A valid interval change shall apply without reboot.

- The configured interval shall be persisted in non-volatile storage.

- Invalid values shall be rejected while preserving the previous valid value.

### FR-02 — AUTO Mode

In AUTO mode, the system shall automatically control temperature, humidity, and reservoir refill according to Section 4.2.

### FR-03 — Heating

When temperature is below Min Temperature, the system shall activate HEAT and keep it active until Target Temperature is reached.

Humidification may operate simultaneously when required.

### FR-04 — Cooling

When temperature is above Max Temperature, the system shall activate COOL and keep it active until Target Temperature is reached.

Humidification may operate simultaneously when required.

### FR-05 — Drying

When humidity is above Max Humidity and neither heating nor cooling is required, the system shall activate DRY.

DRY shall remain active until Target Humidity is reached.

If heating or cooling becomes necessary, DRY shall stop and the appropriate temperature-control mode shall start.

DRY and HUMIDIFY shall never be active simultaneously.

### FR-06 — Humidification

When humidity is below Min Humidity, the system shall activate the humidifier and keep it active until Target Humidity is reached.

Humidification may operate simultaneously with HEAT or COOL.

Humidification shall be blocked when:

- WATER_LOW is active;

- refill is active;

- Safe State is active.

### FR-07 — Climate Control Coordination

The system shall coordinate temperature and humidity control according to the following rules:

- HEAT, COOL, and DRY are mutually exclusive AC modes.

- HEAT and COOL have priority over DRY.

- HUMIDIFY may operate together with HEAT or COOL.

- DRY and HUMIDIFY shall never operate simultaneously.

- All climate-control outputs shall be disabled in Safe State.

### FR-08 — Automatic Reservoir Refill

When LOW Level Sensor is active:

- set WATER_LOW;

- stop and block humidification;

- start reservoir refill.

When FULL Level Sensor becomes active:

- stop reservoir refill;

- clear WATER_LOW;

- set WATER_FULL;

- allow humidification again.

### FR-09 — Humidifier Protection

The humidifier shall not operate when:

- WATER_LOW is active;

- reservoir refill is active;

- Safe State is active.

### FR-10 — Refill Protection

If FULL Level Sensor does not become active within 120 seconds:

- stop refill;

- set REFILL_TIMEOUT;

- block automatic refill;

- publish the fault through MQTT;

- log the fault through diagnostics.

If LOW Level Sensor and FULL Level Sensor are active simultaneously:

- treat the sensor state as invalid;

- stop refill;

- block humidification;

- set WATER_LEVEL_SENSOR_FAULT;

- publish the fault through MQTT;

- log the fault through diagnostics.

### FR-11 — MANUAL Mode

In MANUAL mode, the user may control:

- AC: OFF, HEAT, COOL, DRY;

- humidifier;

- reservoir refill.

Automatic climate-control logic shall not change actuator states in MANUAL mode.

Safety restrictions and Safe State shall override manual commands.

### FR-12 — Parameter Configuration

The user shall be able to remotely configure:

- Min Temperature;

- Max Temperature;

- Min Humidity;

- Max Humidity;

- Measurement Interval;

- WARNING thresholds;

- CRITICAL thresholds.

Each valid parameter change shall:

- apply immediately without reboot;

- be stored in non-volatile memory;

- survive reboot;

- be confirmed through MQTT.

Target Temperature and Target Humidity shall be recalculated automatically after corresponding Min/Max changes.

### FR-13 — Parameter Validation

Each parameter shall be validated independently before it is applied or saved.

If one parameter is invalid:

- reject only that parameter;

- keep all other valid parameters;

- preserve the previous valid value for the rejected parameter;

- notify the user immediately;

- provide the rejection reason.

Validation rules:

- Min < Max

- 0%RH <= Humidity <= 100%RH

- 1 s <= Measurement Interval <= 60 s

- CRITICAL_LOW < WARNING_LOW < CONTROL_MIN < CONTROL_MAX < WARNING_HIGH < CRITICAL_HIGH

- Wrong data type or format shall be rejected.

### FR-14 — Parameter Recommendations

After the user enters one primary control boundary, the system shall suggest a related boundary.

Examples:

- Max Temperature entered → recommend Min Temperature = Max Temperature - 5°C

- Min Temperature entered → recommend Max Temperature = Min Temperature + 5°C

- Max Humidity entered → recommend Min Humidity = Max Humidity - 20%RH

- Min Humidity entered → recommend Max Humidity = Min Humidity + 20%RH

After both boundaries are available, the corresponding target value shall be displayed.

Recommendations:

- shall not be applied automatically;

- require user confirmation;

- shall pass normal validation;

- shall be adjusted to the nearest valid value if necessary.

### FR-15 — Remote Monitoring

The system shall publish through MQTT:

- temperature;

- humidity;

- AUTO / MANUAL;

- current AC mode;

- humidifier state;

- refill state;

- water-level sensor states;

- current configuration;

- Target Temperature;

- Target Humidity;

- NORMAL / WARNING / CRITICAL;

- active faults.

Changes in actuator state, operating mode, or fault state shall be published independently of the next sensor-measurement cycle.

### FR-16 — Remote Control

The system shall accept MQTT commands for:

- switching between AUTO and MANUAL;

- changing configuration;

- manually controlling AC;

- manually controlling humidifier;

- manually controlling refill.

Manual actuator commands shall be accepted only in MANUAL mode.

Commands that violate safety restrictions shall be rejected.

### FR-17 — SHT31 Fault Handling

If SHT31 data is unavailable or unreliable, the system shall enter Safe State.

Safe State shall:

- set AC to OFF;

- turn humidifier OFF;

- stop reservoir refill;

- block automatic actuator control;

- block manual actuator control;

- publish the fault through MQTT;

- log the fault through diagnostics.

Safe State has priority over AUTO and MANUAL.

### FR-18 — Environmental Warning Levels

The system shall classify environmental values as:

- NORMAL;

- WARNING;

- CRITICAL.

WARNING and CRITICAL conditions shall:

- be published through MQTT;

- generate a diagnostic event;

- not automatically trigger Safe State by themselves.

Normal climate regulation shall continue unless another fault requires Safe State.

## 5. Non-Functional Requirements

### NFR-01 — Response Time

After receiving a new valid temperature, humidity, or water-level state, the system shall make the required control decision and update outputs within 1 second.

### NFR-02 — Measurement Accuracy

The system shall support:

- temperature accuracy: ±0.5°C;

- humidity accuracy: ±3%RH.

Software processing shall not introduce additional significant measurement distortion.

### NFR-03 — Startup Time

After power-on or reboot, the system shall reach an operational state within 10 seconds.

All actuators shall remain OFF during initialization.

### NFR-04 — Reboot Recovery

After reboot, the system shall:

- restore the last valid configuration;

- never restore partial or corrupted configuration;

- start with actuators in a safe state;

- initialize sensors;

- restore Wi-Fi and MQTT connectivity;

- re-evaluate current sensor values before activating actuators.

Active actuator states shall not be restored blindly.

### NFR-05 — Network Recovery

Loss of Wi-Fi or MQTT shall not stop local AUTO climate regulation.

The system shall:

- begin reconnect attempts within 5 seconds;

- automatically restore Wi-Fi and MQTT;

- resume telemetry after reconnect;

- not enter Safe State solely because of network loss.

### NFR-06 — MQTT Event Latency

With an active MQTT connection, important state changes shall be published within 2 seconds.

This includes:

- AC mode;

- AUTO / MANUAL;

- humidifier state;

- WATER_LOW;

- WATER_FULL;

- WARNING;

- CRITICAL;

- REFILL_TIMEOUT;

- WATER_LEVEL_SENSOR_FAULT;

- Safe State.

### NFR-07 — Reliability

The system shall operate continuously for 24 hours without:

- hanging;

- unexpected reboot;

- configuration loss;

- uncontrolled actuator state.

Measurement, climate control, and MQTT communication shall remain operational.

### NFR-08 — Configuration Integrity

Configuration shall survive:

- software reboot;

- hardware reboot;

- power cycle.

If configuration is missing or corrupted:

- load default values;

- generate a diagnostic event;

- do not use corrupted values.

Changing one parameter shall not modify unrelated parameters, except automatically calculated dependent target values.

## 6. Critical Scenarios

### CS-01 — Combined Cooling and Humidification

Initial state:

- Temperature: 30°C

- Humidity: 30%RH

- Mode: AUTO

Expected behavior:

- Start COOL because temperature is above Max Temperature.

- Start HUMIDIFY because humidity is below Min Humidity.

- COOL and HUMIDIFY operate simultaneously.

- When temperature reaches Target Temperature, stop COOL.

- If humidity is still below Target Humidity, continue HUMIDIFY.

- When humidity reaches Target Humidity, stop HUMIDIFY.

- Enter IDLE if no other regulation is required.

Related requirements: FR-01, FR-02, FR-04, FR-06, FR-07, NFR-01.

### CS-02 — Low Water During Humidification

- HUMIDIFY is active.

- LOW Level Sensor becomes active.

- Stop humidifier immediately.

- Set WATER_LOW.

- Start reservoir refill.

- FULL Level Sensor becomes active.

- Stop refill.

- Clear WATER_LOW.

- Re-evaluate humidity.

- Resume HUMIDIFY if humidity is still below Target Humidity.

Related requirements: FR-06, FR-08, FR-09, NFR-01.

### CS-03 — Invalid Configuration Parameter

- User changes a valid configuration parameter.

- User enters an invalid value for another parameter.

- Reject only the invalid parameter.

- Preserve all previously accepted valid values.

- Preserve the previous valid value for the rejected parameter.

- Do not persist the invalid value.

- Return an immediate validation response.

Related requirements: FR-12, FR-13, FR-14, NFR-08.

### CS-04 — SHT31 Failure During Active Regulation

- Climate regulation is active.

- SHT31 stops responding or returns invalid data.

- Enter Safe State.

- Turn AC OFF.

- Turn humidifier OFF.

- Stop refill.

- Block manual and automatic actuator control.

- Publish fault through MQTT.

- Log diagnostic event.

Related requirements: FR-17, NFR-01, NFR-06.

### CS-05 — Wi-Fi / MQTT Loss

- Device operates in AUTO mode.

- Wi-Fi or MQTT connection is lost.

- Local climate control continues.

- Device starts reconnect attempts.

- Connection is restored automatically.

- Current actual state is published after reconnect.

Related requirements: FR-01, FR-02, FR-15, NFR-05, NFR-06.

### CS-06 — Reboot During Active Regulation

- HEAT, COOL, DRY, or HUMIDIFY is active.

- Device reboots.

- All actuators remain OFF during initialization.

- Restore the last valid configuration.

- Initialize sensors and water-level inputs.

- Read current environmental state.

- Re-evaluate required actuator states.

- Do not blindly restore the actuator state that was active before reboot.

Related requirements: FR-01, FR-02, FR-12, NFR-03, NFR-04, NFR-08.

### CS-07 — Abnormally Fast Temperature Rise

A thermal anomaly is detected when:

- Temperature increases by at least 10°C within 60 seconds; or

- Temperature reaches at least 45°C.

Expected behavior:

- Classify the event as THERMAL_ANOMALY.

- Turn AC OFF.

- Turn humidifier OFF.

- Stop refill.

- Block automatic and manual actuator control.

- Publish a critical MQTT event.

- Log the diagnostic event.

THERMAL_ANOMALY is treated as a safety condition and has higher priority than normal climate regulation.

This scenario does not represent fire detection, and the device is not a fire-alarm or fire-suppression system.